# Safety Driver Training and Responsibilities

- The Safety Driver ensures the Safe Operation of the Vehicle

- To ensure the safety of the people in and around the vehicle, the vehicle itself and the facility

Make sure:

  - Seat belts are worn by you and the Experimenter

  - All doors are closed and windows are open, so everyone can hear each other

- Your visibility in the vehicle is restricted, so you must have a Safety Look-Out any time the vehicle is to move

- When the Experimenter is ready to test,  you shout “CLEAR”

- This is your Safety Look-Out’s indication to assist you in ensuring:

  - The wheel chocks are removed

  - The vehicle charging cable is unplugged

  - There is nothing in the path of the vehicle

  - Everyone is clear of the vehicle

  - Everyone around is paying attention

- The Safety Look-Out will then reply by shouting “READY”

- When you are prepared, you will respond by shouting “MOVING”

Then you shall:

  - Be ready to take control of the vehicle when the autonomous driving fails

  - Maintain alertness without being distracted by the passenger’s testing

  - Keep aware of any people moving around the vehicle

- If at any time you think safety may be compromised, take control of the vehicle by operating the break or steering wheel – this will disengage the computer control

- Your Safety Look-Out will be watching the area, listen for them to YELL “STOP”

- At any time the Safety Look-Out or Experimenter may press one of the of the Emergency Buttons which will disengage the computer, however it is still your responsibility to take control and stop the vehicle

- Upon normal completion of an experiment you shout “OK” then people can approach vehicle to discuss issues with the experiment and troubleshoot

- The next experiment starts with you shouting “CLEAR”